Print

Step 1

Check if the usb/lpx detected

/dev/usb/lpx

Step 2

Then check the rights

We get crw--rw—1 root ip 180 …something

But we need like the below
pos@pos-PS-3616:~$ ls -l /dev/usb/

total 0

crw------- 1 root root 180, 0 Jul  3 13:24 hiddev0

crw-rw-rw- 1 root lp   180, 1 Jul  3 13:24 lp1

pos@pos-PS-3616:~$

—-------For getting full printer info execute this :[ udevadm info --attribute-walk --name=/dev/usb/lp1 ]

In order to make the rights like that Add this

os@pos-PS-3616:~$

pos@pos-PS-3616:~$ cat /etc/udev/rules.d/99-rongta-printer.rules

KERNEL=="lp[0-9]*", SUBSYSTEMS=="usb", ATTRS{idVendor}=="0fe6", ATTRS{idProduct}=="811e", MODE="0666"

pos@pos-PS-3616:~$

Step 3

Execute the following commands
sudo udevadm control --reload-rules

sudo udevadm trigger

Step 4

Unplug the printer  wait 5 second check the /usb/lpx

Step 5
Reconnect the printer check the usb/lpx

Step 6
Now we will get the rights perfectly

Step 7

Add the script in any folder 
// printer-server.js

const express = require('express');

const fs = require('fs');

const cors = require('cors');

const app = express();

const PORT = 9100;

const PRINTER_DEVICE = '/dev/usb/lp1'; // Use correct lpX device

app.use(cors());

app.use(express.json());

app.post('/print', (req, res) => {

const { name, employeeId, action } = req.body;

const now = new Date().toLocaleString('en-IN'); // Indian locale format

const receipt = `

\x1b@                      // Initialize printer

\x1b!\x38                  // Bold + double height/width

${action.toUpperCase()} SUCCESS\n

\x1b!\x00                  // Reset text style

Name: ${name}

ID: ${employeeId}

Time: ${now}

------------------------

\x1b\x64\x03               // Feed 3 lines

\x1dV\x41\x03              // Full cut with 3-line feed

`.replace(/^\s+/, ''); // Remove indentation on first line only

try {

fs.writeFileSync(PRINTER_DEVICE, receipt);

console.log(`🖨️ Printed receipt for ${employeeId} with cut`);

res.status(200).send('Printed successfully');

} catch (err) {

console.error('‼️ Print failed:', err.message);

res.status(500).send('Printing failed');

}

});

app.listen(PORT, () => {

console.log(`🖨️ Print server running at http://localhost:${PORT}`);

});

Step 8

Run this script check if its working or not

ShutDown

Step 1

Create a folder and add this script specially note the path

const express = require('express');

const { exec } = require('child_process');

const app = express();

const PORT = 3000;

app.post('/shutdown', (req, res) => {

console.log('🛑 Shutdown triggered from GUI');

res.status(200).send('Shutting down...');

// Shutdown after a brief delay to allow response to complete

setTimeout(() => {

exec('shutdown now', (error, stdout, stderr) => {

if (error) {

console.error(`‼️ Shutdown error: ${error.message}`);

return;

}

console.log('🛑 System is shutting down');

});

}, 1000);

});

app.listen(PORT, () => {

console.log(`🛑 Shutdown server running at http://localhost:${PORT}`);

});

Automation of Scripts

pos@pos-PS-3616:~$ cat /etc/systemd/system/shutdown-server.service

[Unit]

Description=Shutdown Server (Node.js)

After=network.target

[Service]

ExecStart=/home/pos/.nvm/versions/node/v22.17.0/bin/node /home/pos/shutdown/shutdown-server.js

Restart=always

User=pos

Environment=PATH=/home/pos/.nvm/versions/node/v22.17.0/bin:/usr/bin:/bin

Environment=NODE_ENV=production

WorkingDirectory=/home/pos/shutdown

[Install]

WantedBy=multi-user.target

Sudo systemctl daemonreexec service name

Sudo systemctl daemonreload service name

Sudo systemctl enable service name

Sudo systemctl start service name

Sudo systemctl status service name

pos@pos-PS-3616:~$ cat /etc/systemd/system/printer-server.service

[Unit]

Description=Printer Server (Node.js + NVM)

After=network.target

[Service]

Type=simple

ExecStart=/home/pos/.nvm/versions/node/v22.17.0/bin/node /home/pos/print/print_script.js

WorkingDirectory=/home/pos/print

Restart=always

User=pos

Environment=NODE_ENV=production

Environment=PATH=/home/pos/.nvm/versions/node/v22.17.0/bin:/usr/bin:/bin

[Install]

WantedBy=multi-user.target

Repeat the sudo systemctl commands