switch5b1e79#

switch5b1e79#

switch5b1e79#show running-config

config-file-header

switch5b1e79

v1.4.11.4 / R800_NIK_1_4_219_025

CLI v1.0

set system mode switch

file SSD indicator encrypted

@

ssd-control-start

ssd config

ssd file passphrase control unrestricted

no ssd file integrity control

ssd-control-end cb0a3fdb1f3a1af4e4430033719968c0

!

vlan database

vlan 20,102-104

exit

voice vlan oui-table add 0001e3 Siemens_AG_phone________

voice vlan oui-table add 00036b Cisco_phone_____________

voice vlan oui-table add 00096e Avaya___________________

voice vlan oui-table add 000fe2 H3C_Aolynk______________

voice vlan oui-table add 0060b9 Philips_and_NEC_AG_phone

voice vlan oui-table add 00d01e Pingtel_phone___________

voice vlan oui-table add 00e075 Polycom/Veritel_phone___

voice vlan oui-table add 00e0bb 3Com_phone______________

dot1x system-auth-control

hostname switch5b1e79

encrypted radius-server key WSX6YFwgD28kkPTEDWczuxRpCZGlUr0LVvgN3nS+s/I=

radius-server host 172.20.100.104

aaa accounting dot1x start-stop group radius

username admin password encrypted 27a91d311ee0d8bf49cdb75dabb6cf33d7565c25 privilege 15

ip ssh password-auth

snmp-server server

snmp-server community public ro view Default

snmp-server community private rw view Default

snmp-server host 172.20.100.104 traps version 2c public

snmp-server source-interface traps vlan 20

snmp-server source-interface informs vlan 20

!

interface vlan 20

name management

ip address 172.20.100.8 255.255.255.0

no ip address dhcp

!

interface vlan 102

name registration

!

interface vlan 103

name isolation

!

interface vlan 104

name normal

!

interface gigabitethernet1

switchport trunk allowed vlan add 20,102-104

!

interface gigabitethernet2

dot1x host-mode multi-sessions

dot1x reauthentication

dot1x timeout quiet-period 10

dot1x timeout reauth-period 300

dot1x timeout server-timeout 5

dot1x timeout supp-timeout 3

dot1x authentication mac

dot1x radius-attributes vlan

dot1x port-control auto

spanning-tree portfast

switchport mode access

switchport access vlan 102

!

interface gigabitethernet5

switchport mode access

switchport access vlan 20

!

interface gigabitethernet6

switchport trunk allowed vlan add 20,102-104

!

interface gigabitethernet7

switchport mode access

switchport access vlan 20

!

interface gigabitethernet8

switchport trunk allowed vlan add 102-104

!

interface gigabitethernet12

switchport mode access

switchport access vlan 20

!

interface gigabitethernet13

switchport mode access

switchport access vlan 20

!

interface gigabitethernet14

switchport trunk allowed vlan add 102-104

!

interface gigabitethernet19

switchport mode access

switchport access vlan 102

!

interface gigabitethernet20

switchport trunk allowed vlan add 102-104

!

interface gigabitethernet21

switchport mode access

switchport access vlan 20

!

interface gigabitethernet22

switchport trunk allowed vlan add 102-104

!

interface gigabitethernet25

switchport mode access

switchport access vlan 104

!

interface gigabitethernet26

switchport mode access

switchport access vlan 20

!

interface gigabitethernet27

switchport mode access

switchport access vlan 20

!

interface gigabitethernet29

dot1x host-mode multi-sessions

dot1x reauthentication

dot1x timeout quiet-period 10

dot1x timeout reauth-period 300

dot1x timeout server-timeout 5

dot1x timeout supp-timeout 3

dot1x authentication mac

dot1x radius-attributes vlan

dot1x port-control auto

spanning-tree portfast

switchport mode general

switchport general allowed vlan add 102 untagged

switchport general pvid 102

!

interface gigabitethernet30

switchport mode access

switchport access vlan 102

!

interface gigabitethernet42

switchport mode access

switchport access vlan 20

!

interface gigabitethernet48

switchport trunk allowed vlan add 20,102-104

switchport default-vlan tagged

!

exit

switch5b1e79#